PROPOSAL IN RESPONSE TO

Industrial IoT Edge Gateway Platform

Submitted to: IndustraTech Manufacturing Solutions

| RFP Reference Number | RFP-2024-EMB-003 |
| --- | --- |
| Submitted By (Vendor) | EdgeWave Systems Pvt. Ltd. |
| Submitted To (Client) | IndustraTech Manufacturing Solutions |
| Submission Date | 01 August 2024 |
| Proposed Engagement Duration | 12 months |
| Total Proposed Cost | USD 195,000 |
| Proposal Validity | 90 days from submission date |
| Point of Contact | [Vendor Contact Name, Email, Phone] |

CONFIDENTIAL — FOR CLIENT USE ONLY

# 1. Executive Summary

EdgeWave Systems proposes a turnkey Industrial IoT Edge Gateway solution for IndustraTech's factory floor digitalization initiative. Our platform is built on the NXP i.MX 8M Mini SoC running a hardened Linux (Yocto) image, providing multi-protocol ingestion (Modbus RTU/TCP, PROFINET, OPC-UA), local edge analytics via ONNX Runtime, and secure cloud telemetry to AWS IoT Core via MQTT over TLS 1.3. The gateway is designed for DIN-rail mounting, -25°C to +70°C operation, and integrates hardware security via TPM 2.0 for device identity and certificate management.

✔  This proposal is submitted in full compliance with the requirements set forth in RFP-2024-EMB-003.

# 2. Company Profile

Company Name: EdgeWave Systems Pvt. Ltd.

Headquarters: [City, Country]

Founded: [Year]

Team Size: [X] engineers (embedded firmware, hardware, test)

Certifications: [ISO 9001:2015 / ISO 13485:2016 / CMMI Level 3 — as applicable]

Relevant Domains: [Automotive / Medical / Industrial / Defense / Consumer]

Key credentials relevant to this engagement:

- Deployed 200+ industrial IoT gateways across automotive, food processing, and pharmaceutical sectors

- Certified AWS IoT Competency Partner with pre-validated Greengrass v2 reference architecture

- In-house PROFINET conformance test lab — no outsourcing of protocol certification testing

- Full Yocto BSP development capability; own maintained meta-layer for NXP i.MX family

# 3. Technical Approach

## 3.1 Solution Architecture Overview

Our proposed architecture for the Industrial IoT Edge Gateway Platform is described below. All components have been selected based on the requirements stated in RFP-2024-EMB-003 and validated against our prior project baselines.

| Parameter | Proposed Solution |
| --- | --- |
| SoC / SOM | NXP i.MX 8M Mini (4x Cortex-A53 + 1x Cortex-M4F), SMARC 2.1 |
| OS / Distribution | Yocto Kirkstone, Linux 6.1 PREEMPT_RT, systemd, Docker |
| Industrial Protocols | Modbus RTU/TCP, PROFINET RT, OPC-UA (open62541), BACnet |
| Connectivity | Dual GbE, 4G LTE (Quectel EC25), Wi-Fi 5 (optional), USB 3.0 |
| Security | TPM 2.0 (Infineon SLB 9672), Secure Boot, dm-crypt, mTLS |
| Cloud Platform | AWS IoT Greengrass v2, MQTT, InfluxDB + Grafana |
| Edge AI | ONNX Runtime 1.16, TFLite 2.13, ARM NN delegate |

## 3.2 Implementation Methodology

Our development approach follows an incremental, milestone-gated model with continuous integration and automated regression testing. Key methodology elements:

- Hardware: custom carrier board around NXP i.MX 8M Mini SOM (SMARC 2.1), with 4x RS-485 ports, 2x Gigabit Ethernet, 4G LTE modem (Quectel EC25), isolated digital I/O (8 DI / 8 DO), and TPM 2.0 (Infineon SLB 9672)

- OS: Yocto Project Kirkstone-based image with real-time kernel patch (PREEMPT_RT) for deterministic Modbus polling

- Protocol translation: open62541 OPC-UA server bridging Modbus RTU/TCP and PROFINET device data to unified OPC-UA information model

- Edge ML inference: ONNX Runtime 1.16 with anomaly detection models (vibration, temperature) running at the edge with <50 ms inference latency

- Cloud connectivity: AWS IoT Greengrass v2 for component lifecycle management; MQTT telemetry with TLS 1.3 and X.509 mutual auth

- Remote management: Balena Fleet for OTA OS/container updates; device health dashboard via Grafana + InfluxDB on AWS

- Security: TPM 2.0 device identity, secure boot chain (U-Boot + IMA/EVM), encrypted rootfs (dm-crypt/LUKS)

## 3.3 Tools & Development Environment

| Category | Tool / Platform | Purpose |
| --- | --- | --- |
| IDE / Toolchain | [Primary IDE & GCC/LLVM version] | Code editing, compilation, debugging |
| Version Control | Git (GitLab self-hosted) | Source control, MR-based code review |
| CI/CD | GitLab CI + custom runners | Build, static analysis, unit test on every commit |
| Static Analysis | [PC-lint / Polyspace / Coverity] | MISRA-C / coding standard enforcement |
| Unit Testing | [Unity / GoogleTest / CppUTest] | Automated unit & integration test |
| Requirements | [JIRA + Polarion / IBM DOORS] | Requirements traceability matrix (RTM) |
| HIL/SIL | [dSPACE / X-Plane / custom harness] | Hardware & software in-the-loop testing |

# 4. Project Plan & Schedule

## 4.1 Milestone Schedule

| # | Milestone | Target |
| --- | --- | --- |
| M1 | Architecture Review & Hardware Design Kickoff | Week 2 |
| M2 | SOM Carrier Board Rev A & Bringup | Week 10 |
| M3 | Yocto BSP & Secure Boot Operational | Week 16 |
| M4 | Protocol Stack Integration (Modbus, OPC-UA, PROFINET) | Week 24 |
| M5 | Cloud Connectivity & OTA Pipelines Live | Week 32 |
| M6 | Edge ML Integration & Factory Pilot Deployment | Week 42 |
| M7 | Hardening, Documentation & Production Handover | Week 48 |

## 4.2 Risk Management

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| Component supply chain delay (long-lead semiconductors) | Medium | High | Place orders at M1; dual-source critical ICs identified |
| Requirements creep post architecture freeze | High | Medium | Formal change control via signed ECO; impact assessed before action |
| Third-party IP / stack integration delays | Low | Medium | Pre-evaluated in proposal; fallback open-source options identified |
| Regulatory/certification timeline extension | Medium | High | Early engagement with test lab; pre-compliance checks at M4 |
| Key personnel availability | Low | High | Named resources contractually committed; documented handover plan |

# 5. Proposed Team

| Role | Name | Experience | Allocation |
| --- | --- | --- | --- |
| Engagement / Program Manager | [Name] | [X] yrs PM in embedded programs | 20% |
| Lead Firmware Engineer | [Name] | [X] yrs, [relevant MCU/domain] | 100% |
| Hardware / PCB Engineer | [Name] | [X] yrs, [CAD tool] | 60% |
| System / Safety Engineer | [Name] | [X] yrs, [standard] | 50% |
| Test & Verification Engineer | [Name] | [X] yrs, HIL/SIL | 80% |
| Software QA Engineer | [Name] | [X] yrs, [QMS experience] | 40% |

All named personnel are available for this engagement from project kickoff. Any substitution requires written client approval.

# 6. Past Performance & References

| Client | Project | Value | Outcome | Reference Contact |
| --- | --- | --- | --- | --- |
| [Client A] | [Similar project description] | [USD X] | On-time, certified delivery | [Name, email] |
| [Client B] | [Similar project description] | [USD X] | Zero field defects post-deployment | [Name, email] |
| [Client C] | [Similar project description] | [USD X] | Regulatory approval achieved | [Name, email] |

# 7. Pricing & Cost Breakdown

✔  All prices are fixed-price per milestone. T&M rates available for change orders: Senior Engineer USD [X]/hr, Engineer USD [X]/hr.

| Phase / Deliverable | Pricing Model | Cost (USD) |
| --- | --- | --- |
| Hardware Design & Carrier Board Prototypes (Rev A+B) | Lump Sum | USD 38,000 |
| Yocto BSP, Kernel & Secure Boot | Lump Sum | USD 28,000 |
| Industrial Protocol Stack (Modbus, OPC-UA, PROFINET) | Lump Sum | USD 35,000 |
| Cloud Connectivity & Fleet Management | Lump Sum | USD 30,000 |
| Edge ML Pipeline & Model Integration | Lump Sum | USD 28,000 |
| Security Hardening (TPM, dm-crypt, IMA) | Lump Sum | USD 18,000 |
| Testing, Pilot Deployment & Documentation | Lump Sum | USD 18,000 |
| TOTAL |  | USD 195,000 |

Payment milestones aligned with M1–M7 above. 20% advance on contract signing; remainder milestone-gated.

# 8. Assumptions & Exclusions

## 8.1 Assumptions

- Client will provide detailed technical requirements document within 5 business days of kickoff

- Client hardware (EVK / DVT boards) will be available to vendor within 2 weeks of project start

- Client will designate a named technical point of contact available for at least 4 hours/week

- All third-party tool licenses (if client-mandated) will be provided by client

- Travel costs (if on-site visits required) will be billed at actuals with prior written approval

## 8.2 Exclusions

- Mechanical / enclosure design and fabrication

- Mass production PCB assembly or component procurement beyond prototypes

- Regulatory certification filing fees and test lab charges (budgeted separately)

- Cloud backend infrastructure setup and ongoing hosting costs

- Mobile / companion app development (API/GATT profile spec only)

# 9. Authorization & Acceptance

The undersigned authorized representative of EdgeWave Systems Pvt. Ltd. certifies that the information in this proposal is accurate, complete, and that the company is fully capable of delivering the described solution within the stated timeline and budget.

| Field | Vendor Representative | Client Acceptance |
| --- | --- | --- |
| Name |  |  |
| Title |  |  |
| Signature |  |  |
| Date |  |  |
| Company | EdgeWave Systems Pvt. Ltd. | IndustraTech Manufacturing Solutions |

— END OF PROPOSAL | RFP-2024-EMB-003 | EdgeWave Systems Pvt. Ltd. —